# TR-55 TIME-OF-CONCENTRATION WORKSHEET

# Route:        Project No.:        Location:

Designer:       Date:

Checked By:        Date:

# Present   Developed

# Tc   Tt   Through subarea

# A map, schematic, or flow-segments description is attached.

| Sheet Flow, applies to Tc only Segment ID |  |  |
| --- | --- | --- |
| 1. Surface description, see Figure 202-2B |  |  |
| 2. Manning’s roughness coeff., n, see Figure 202-2B |  |  |
| 3. Flow length, L (total L ≤ 100 ft) | ft | ft |
| 4. Two-year 24-h rainfall | in. | in. |
| 5. Land slope, s | ft / ft | ft / ft |
| 6. Tt=0.007nL0.8P20.5s0.4 | h | h |

Total Tt in line 6 for both segments =       h

| Shallow Concentrated Flow Segment ID |  |  |
| --- | --- | --- |
| 7. Surface description, paved or unpaved |  |  |
| 8. Flow length, L | ft | ft |
| 9. Watercourse slope, s | ft / ft | ft / ft |
| 10. Average velocity, V, see Figure 202-2D | ft / s | ft / s |
| 11. Tt=L3600V | h | h |

Total Tt in line 11 for both segments =       h

| Channel Flow Segment ID |  |  |
| --- | --- | --- |
| 12. Cross-sectional flow area, a | ft2 |  |
| 13. Wetted perimeter, Pw | ft | ft |
| 14. Hydraulic radius, r = a/Pw | ft | ft |
| 15. Channel slope, s | ft / ft | ft / ft |
| 16. Manning’s roughness coeff., n, see Figure 202-2C |  |  |
| 17. V=1.49r2/3s1/2n | ft / s | ft / s |
| 18. Flow length, L | ft | ft |
| 19. Tt=L3600V | h | h |

Total Tt in line 19 for both segments =       h

20. Add Tt in lines 6, 11, and 19 to get watershed or subarea Tc or Tt =       h