PRESSURE DIAGNOSTIC GUIDED DUCT SEALING/AIR SEALING OF THE BUILDING SHELL

All materials used in provision of services must meet or exceed 10 CFR 440 Appendix A: “Standards for Weatherization Services”

Contractor is responsible for providing air sealing/duct sealing measures in accordance with BPI or other state directed minimum standards designed to ensure the health and safety of the occupants while improving energy efficiency.

Duct leakage must be documented before and after duct sealing occurs. This must be done using approved pressure diagnostics measurement equipment such as a Blower Door or equal to prioritize work to ensure largest leaks and highest pressures are sealed first. Following BPI or State of 		 Standards for Minimum Health and Safety Requirements at all times. If the measured whole house CFM50 is below minimum airflow standards set by BPI or State of 		standards then mechanical ventilation must be installed. If combustion appliances are present, contractor must perform a Combustion Safety test and CAZ tests both before and after work completed and  report results to ensure that minimum safety requirements are met as outlined in BPI Building Analyst Technical Standards/ Combustion Safety Test Action Levels and CAZ Depressurization or State of 			Limits. These tests are required and must be included as part of all duct sealing/air sealing measures. Contractor should include cost to perform these tests as part of the standard measures below.

Contractor is responsible for leaving the job site clean, hauling away job debris and existing equipment (if applicable) and for properly disposing of existing equipment to meet EPA regulations.

Project commencement and completion must be accomplished within _________ working days of notice to proceed.

| MEASURES |  |
| --- | --- |
| Duct Sealing/Air Sealing | Per Unit Cost |
| Properly seal all duct connections including drywall to boot connections and plenum to air handler both supply and return side. All work must be done using approved pressure diagnostics measurement equipment such as a Blower Door or equal. | $ |
| Install Air Barrier (per square foot installed) (using duct board or WAP approved equal) | $ |
| 90 cmf Fan | $ |

Signature						Date