HVAC SPECIFICATIONS AND PRICING QUOTE

All materials used in provision of services must meet or exceed 10 CFR 440

Appendix A: “Standards for Weatherization Services”

All new HVAC installations are to be performed in accordance with the most recent Universal Mechanical Code and/or local building code.

Permits will be required as applicable by local building code. It will be the responsibility of the Contractor to ensure that necessary permits have been secured prior to starting work. This cost must be included in each line item price below.

The Contractor shall be responsible for ensuring properly sized gas line is supplying the heater and shall inform agency in the event that existing line is improperly sized. The Contractor shall leave all literature on the new units with the client and shall also inform the client of proper care and maintenance required on the new units.

If required by system, all venting and combustion air shall be installed in accordance with AGA and GAMA specification. New gas forced air furnaces shall have pilotless ignition.

All new unit installations shall include new thermostat.

All duct work connections and holes shall be sealed, on all sides, with a non-toxic, Department of Energy industry approved, mastic duct sealant applied per manufacturers specifications.

New conduit or gas pipe installed through the roof shall include new properly sized roof jack with 3-course seal at the roof.

All installation estimates shall include removal and proper disposal of existing equipment, recovery of refrigerant from existing system as required by law.

Contractor is responsible for leaving the job site clean, hauling away job debris, and existing equipment (if applicable) and for properly disposing of existing equipment to meet EPA regulations.

Project commencement and completion must be accomplished within _________ working days of notice to proceed.

Please list the Manufacturer brand name and model you will be using for the scope of work listed below.

| Heat Pump Unit | Brand: |
| --- | --- |
| Gas Package Unit | Brand: |
| Evaporative Cooler | Brand: |
| Furnace | Brand: |

All Contractor bid amounts below for installs of package units to include new elbow or roof curb, t-stat and stand, disconnect and fuses and condensate drain. All installs of split systems to include new line set, t-stat, condenser pad, disconnect, fuses and gas line if applicable. ALL tie–ins to ductwork to be sealed with a U.L approved duct mastic.  Contractor is to ensure that all units are properly charged to the manufacturer’s specification.  Contractor is responsible for properly sizing unit using a Manual J or other heat load calculation.

REPLACEMENT:  Contractor must include permit fees, sales taxes, labor and materials to complete project.

| ITEM 1 |  |
| --- | --- |
| Description Of Scope Of Work | Unit Price |
| Removal of forced air furnace in closet and evaporative cooler on roof and replace with new 13-SEER Gas Pac unit on roof unit price should include sealing existing furnace, capping gas or electric and running new gas and electrical lines to roof for the following sizes: | CONTRACTOR’S BID AMOUNT $ |

| ITEM 2 |  |
| --- | --- |
| Description Of Scope Of Work | Unit Price |
| Replace existing split system heat pump with new 13-SEER split heat pump | CONTRACTOR’S BID AMOUNT $ |

Initials

NEW DUCT SYSTEM

| ITEM 3: New Return |  |  |
| --- | --- | --- |
| Description and Scope of Work |  | Unit Price |
| a. | Rigid | $ |
| b. | Flex | $ |

| ITEM 4: New Supply Drop (provide a per drop price here) |  |  |
| --- | --- | --- |
| Description and Scope of Work |  | Unit Price |
| a. | Rigid | $ |
| b. | Flex | $ |

| ITEM 5: Jumper Duct System (per room price) | $ |
| --- | --- |

| ITEM 6: Install insulation on Ducts and Heating Pipes (per lineal foot to an R- | $ |
| --- | --- |

================================================================================

REPAIRS:  Provide labor rate per hour for HVAC repairs.

| HVAC REPAIRS |  |
| --- | --- |
| Labor Rate Per Hour | $ |

Materials will be reimbursed at actual cost with an allowance for handling of materials at a percentage of actual cost.  Contractor will be required to submit the actual receipt with all reimbursement requests.

Handling Cost % of actual materials cost.

Signature						Date