WEATHERIZATION PRICING QUOTE

All materials used in provision of services must meet or exceed 10 CFR 440

Appendix A: “Standards for Weatherization Services”

Detailed below is the work to be performed on an individual house.

Contractor is responsible for leaving the job site clean, hauling away job debris, and existing equipment (if applicable) and for properly disposing of existing equipment or debris to meet EPA regulations.

Project commencement and completion must be accomplished within _________ working days of notice to proceed.  Contractor is to notify agency as soon as project is completed for inspection.

Contractor must include permit fees (if applicable), sales taxes, labor and materials to complete project.

| Description of Work | Total Cost |
| --- | --- |
| Seal all duct connections including drywall to boot connections and plenum to air handler both supply and return side. |  |
| Insulate 1,500 sq ft attic with blown cellulose; bring to an R-50 from an R-18 and add 2- 14” X 24” louvered vents to gable ends for attic ventilation. Seal all by-pass and install non-flammable shielding around any heat producing devices. |  |
| Insulate 1,500 sq ft floor with fiberglass kraft faced batt R-30 X 16” OC, well secured with twine, wood strips or wire insulation supports and install black 6 mill polyethylene ground moisture barrier. |  |
| Air filters – replace existing air filter and clean air handler area before installing with new all natural fiber washable and re-useable 20” x 20” x 1” |  |
| Install two windows - Insulated, Low-E, White Vinyl 36" x 60" Horizontal slider windows and replace interior trim and casing lumber with new. Windows meet egress code. (Note: LSW applies to this project). |  |
| Install one 1 ¾” exterior wood solid core door 36” x 80”, standard jam with new 2 ¾” keyed entrance lockset, weather-strip kit, oak threshold and door shoe. (Note: LWS applies to this project). |  |
| Install one 40 gallon gas water heater to include properly sized water and gas lines, combustion air, venting, pressure relief value & electrical hook-up to meet code requirements |  |
| Install 5- 18 watt CFL bulbs |  |
| TOTAL COST |  |

Signature						Date