Huntington County Health Department 1330 S. Jefferson Street Huntington, IN 46750

Office (260) 358-4831 Fax (260) 358-4899

www.huntington.in.us

# Well Location Permit Process

Huntington County Ordinance 2024-03 requires a well location permit to be issued before construction begins for any water well. To obtain a well permit, the following information must be submitted to this Department:

- A complete application

- A property site plan showing the layout of all structures (existing and proposed), property lines, location of the proposed new well, and location of the on-site sewage system components.

  - Dwellings or other access-inhibiting structures must be 10’ or more from the well.

  - All parts of the on-site sewage system shall be at least 50’ from the well serving a one- or two-family dwelling. For commercial and all other wells, the minimum separation distance between the well and the on-site sewage system is 100 feet.

  - Plans must show all property lines and the applicable distance to the proposed well.

    - If the property is located within or adjacent to an agricultural zoning district, the well must be at least 70’ from the property line(s).

    - For all other zoning districts, the well must be 50’ or more from property lines.

- At completion of drilling, a copy of the well log must be sent to the Huntington County Health Department.

- A water sample must be tested after drilling. The water laboratory (or water laboratories) shall test for, and report on, the presence or absence of

  - total coliform bacteria;

  - E. coli;

  - nitrate concentrations measured as nitrogen;

  - arsenic

  - chlorine residual.

A permit fee of $15.00 is due upon submission of the application.