Location:      				Project Number:

Light Source Type:      			IES Light Distribution Type:

---------------------------------------------------------------------------------------------------------------------

Service Cost per Year for Luminaire Replacement or Partial Modernization

=  Annual Energy Cost + Annual Maintenance Cost + (Installation Cost/Warranty Period)

------------------------------------------------------------------------------------------------------------

Annual Energy Cost

= [(No. of Luminaires x Wattage per Luminaire x Operational Hours)÷1000] x Unit Cost of Electricity

No. of Roadway Luminaires =       (1)

Roadway Luminaire Wattage =       (2)

No. of High Mast Luminaires =       (3)

High Mast Luminaire Wattage =       (4)

Operational Hours = 4380

Unit Cost of Electricity =       (5)  ($0.10 per kWh or location specific rate)

Annual Energy Cost =

[{(      x      ) + (       x       )}  x 4380 ÷ 1000] x       =

(1)                 (2) 	    (3)                (4)  					(5)   	  (A)

____________________________________________________________________________

Annual Maintenance Cost

= No. of Luminaires x Maintenance Cost per Luminaire

No. of Roadway Luminaires =       (1)

Maintenance Cost per Roadway Luminaire,

for 250W or 400W HPS  =  $60  (6)

for other light source types =       (7) (per manufacturer’s info)

No. of High Mast Luminaires =       (3)

Maintenance Cost per High Mast Luminaire,

for 1000W = $105 (8)

for other light source types =       (9) (per manufacturer’s info)

Annual Maintenance Cost = {(      x      ) + (      x      )} =

(1)            (6)  or (7)              (3)            (8)  or (9)                (B)

_____________________________________________________________________________

Installation Costs

= No. of Luminaires x Furnish & Install Cost per Luminaire

No. of Roadway Luminaires =       (1)

Furnish/Install Cost per Roadway Luminaire =       (10) (for HPS use bid history)

Service Life =  years (11)

No. of High Mast Luminaires =       (3)

Furnish/Install Cost per High Mast Luminaire =       (12) (for HPS use bid history)

Service Life=  years (13)

Installation Cost = [(      x       ÷      ) + (      x       ÷      )/=

(1)                 (10)              (11)                    (3)               (12)              (13)                  (C)

---------------------------------------------------------------------------------------------------------------------

Service Cost per Year =       +       +       =

(A) 	    (B) 	        (C)