Location:      				Project Number:

System Configuration:      	Pole Spacing:      	Mounting Height:

Light Source Type:      		IES Light Distribution Type:

--------------------------------------------------------------------------------------------------------------------

Service Cost per Year for New Installations or Complete Modernization

=  Annual Energy Cost + Annual Maintenance Cost + (Installation Cost/Warranty or Service Period)

---------------------------------------------------------------------------------------------------------------------

Annual Energy Cost

= [(No. of Luminaires x Wattage per Luminaire x Operational Hours)÷1000] x Unit Cost of Electricity

No. of Luminaires =       (1)

Luminaire Wattage =       (2)

No. of High Mast Luminaires =       (3)

High Mast Luminaire Wattage =       (4)

Operational Hours = 4380

Unit Cost of Electricity =       (5)  ($0.10 per kWh or location specific rate)

Annual Energy Cost =

[{(      x      ) + (       x       )}  x 4380 ÷ 1000] x       =

(1)                 (2) 	    (3)                (4)  					(5)   	  (A)

_________________________________________________________________________________

Annual Maintenance Cost

= No. of Luminaires x Maintenance Cost per Luminaire

No. of Luminaires =       (1)

Maintenance Cost per Luminaire,

for HPS  = $60  (6)

for other light source types =       (7) (per manufacturer’s info)

No. of High Mast Luminaires =       (3)

Maintenance Cost per High Mast Luminaire,

for 1000W = $105 (8)

for other light source types =       (9) (per manufacturer’s info)

Annual Maintenance Cost = {(      x      ) + (      x      )} =

(1)            (6)  or (7)              (3)            (8)  or (9)                (B)

_________________________________________________________________________________

Installation Costs

= [Cost of Luminaires ÷ Warranty Period] +

[Cost of Poles/Foundations ÷ Pole Service Life] +

+ [Cost of Towers/Foundations ÷ Tower Service Life]

No. of Roadway Luminaires =       (1)

Furnish/Install Cost per Roadway Luminaire =       (10) (per mfr. for non-HPS)

=  years (11)

No. of  Poles Foundations =       (12)

Furnish/Install Cost of Pole =       (13)

Furnish/Install Cost of Pole Foundation =       (14)

Pole Service Life = 20 years

No. of High Mast Luminaires =       (3)

Furnish/Install Cost per High Mast Luminaire =       (15) (per mfr. for non-HPS)

=  years (16)

No. of  High Mast Towers Foundations =       (17)

Furnish/Install Cost of Tower =       (18)

Furnish/Install Cost of Tower Foundation =       (19)

Tower Service Life = 40 years

Installation Cost =

[(      x      ) ÷      ] =       (20)

(1)                (10)                (11)

+	[(      x (      +      )) ÷ 20] =       (21)

(12)                  (13)              (14)

+	[(      x      ) ÷      ] =       (22)

(3)                (15)                (16)

+	[(      x (      +      )) ÷ 40]  =       (23)

(17)               (18)                 (19)

/=       +       +       +      =

(20) 	             (21)               (22)	  (23)	    (C)

-------------------------------------------------------------------------------------------------------------------------------

Service Cost per Year =       +       +       =

(A) 	    (B) 	        (C)