MAINTENANCE OF TRAFFIC

The Standard Specifications are revised as follows:

SECTION 104, BEGIN LINE 137, DELETE AND INSERT AS FOLLOWS:

104.04 Maintenance of Traffic. Unless otherwise provided, the road shall be kept open to all traffic while undergoing improvements. Where so provided on the plans, the traffic may be bypassed over an approved detour route. The detour route markings shall be erected, maintained, and removed by the Contractor. Maintenance of traffic shall be in accordance with the details as shown on the plans or as directed by the Department. In the event If an

alternate plan for maintaining traffic is requested, it shall be submitted in writing as soon as possible for consideration. If approved, the alternate plan shall not increase the cost of maintaining traffic to the Department. The portion of the roadway being used by public traffic shall be kept in such condition that such traffic will be adequately accommodated. Drums in accordance with 801.09 shall be placed at 60 m (200 ft) intervals where drop-offs of greater than 75 mm (3 in.) are adjacent to the shoulder until the aggregate or earth wedge is placed. Temporary approaches to businesses, parking lots, residences, garages, farms, and crossings and intersections with trails, roads, and  streets shall be provided in a safe condition.  All traffic control devices shall be maintained with no additional payment, except as set out in 107.17. Regulatory controls shall not be changed by the Contractor without prior approval from the Department. Regulatory control devices may be relocated

in order to permit necessary construction, providing provided these control devices remain

effective and convey the intended meaning after relocation to a position which complies with the requirements of the MUTCD. After completion of the construction, regulatory control devices which were relocated to facilitate construction shall be permanently installed with no additional payment, in accordance with the plans, or as otherwise directed. Any All traffic control devices damaged, while being moved or handled, shall be

replaced by the Contractor with no additional payment. All other traffic control devices necessary to

maintain safe traffic operations and routings shall not be removed, changed, or relocated, except as authorized. Traffic control devices removed without authorization shall be replaced with no additional payment. The cost of maintaining traffic over the section of road undergoing improvement and the cost of the construction and maintenance of such necessary features as approaches, crossings and intersections shall be included in the contract unit price bid for maintenance of traffic pay items as set out in the Schedule of Pay Items, except as provided in 104.04(a), 104.04(b), 104.04(c), and 107.17.  If performed by the Contractor, the maintenance of such features as the roadway surface, approaches, crossings, and intersections will be paid for in accordance with 105.13.

SECTION 104, BEGIN LINE 176, DELETE AND INSERT AS FOLLOWS:

(b) Maintenance Directed by the Engineer. If special maintenance is directed by the Department for the benefit of the traveling public, payment will be made on the basis of unit prices

or in accordance with 104.03 or 105.13. The Engineer will be the sole judge of work to be classed

as special maintenance whether special maintenance shall be performed.

The Engineer may direct in writing that the Contractor perform general road patching or other repair work to areas within the project limits as part of such special maintenance.  The Contractor will not be required to perform such work on the undisturbed road surface unless specifically directed by the Engineer to do so.

Unless mutually agreed upon in writing, the Contractor will not be required to perform snow plowing, ice control, or other work related to temporary weather conditions.

(c) Two-Way Traffic Between December 1 and April 1.  Except as otherwise expressly provided in the contract, existing Department

maintained roads and other public roads and streets within the project limits of the contract shall

be kept open to two-way traffic between the dates of December 1 and April 1.

Where the surface on an existing road or street is disturbed by the Contractor and the entire depth

of the new surface is not completed prior to December 1, two-way traffic shall be maintained between the above dates on the partially completed new surface or on a temporary surface satisfactory for two-way traffic. Such surfaces shall be maintained between the above dates with no additional payment.  Precautions shall be taken to prevent unnecessary damage to partially completed surfaces.  All portions which become damaged shall be repaired by the Contractor with no additional payment.

Public roads, commercial and private drives, and mailbox approaches which are disturbed during construction activities and operations and on which the surfacing has not been completed, shall be maintained in a

condition satisfactory for use during the time the work is suspended.

SECTION 104, DELETE LINE 208.

44444444444444444444444444444444444444444444444444444444444444444444444444