STEEL SHEET PILING AT RAILROAD GRADE SEPARATION

The Standard Specifications are revised as follows:

SECTION 108, AFTER LINE 169, INSERT AS FOLLOWS:

If temporary steel sheet piling is shown on the plans or is otherwise specified, except where required for railroad protection as described in 206.08, an approved temporary retention system may be used.

SECTION 108, BEGIN LINE 172, DELETE AND INSERT AS FOLLOWS:

and disposed of with no additional payment except as provided herein. If temporary roadbed or flexible asphalt pavement materials are used for embankment construction, such materials will be classified as excavation and paid for at the contract unit price per cubic meter (cubic yard) for the type of excavation shown in the Schedule of Pay Items in accordance with 203.28. No

allowance will be made for overhaul or added haul. If temporary flexible HMA pavement materials are used in the work for subbase, base, approaches, or for new shoulder construction, such materials will be paid for as salvaged road material in accordance with the applicable provisions of 613.05.

Temporary concrete pavement, temporary concrete base, or temporary concrete widening, when no longer required for maintenance of traffic, shall be removed and disposed of in accordance with the applicable provisions requirements of 202.05. Such removal and disposal will be paid for as pavement removal in accordance with 202.13.

SECTION 108, DELETE LINES 186 THROUGH 193.

SECTION 203, BEGIN LINE 1149, DELETE AND INSERT AS FOLLOWS:

compaction of materials in accordance with 203.23. The costs of labor, equipment,

tools overhaul, added haul, and necessary incidentals shall be included in the costs of embankment. The cubic

SECTION 206 AFTER LINE 173, INSERT AS FOLLOWS:

Steel sheet piling may be required for railroad protection as shown on the plans. Sheet piling with a higher section modulus than that specified may be required by the railroad company or by the Contractor's bearing design. Sheet piling shall be cut to 0.3 m (1 ft) below the final ground elevation, and shall be left in place after construction is complete.

SECTION 206, AFTER LINE 237, INSERT AS FOLLOWS:

Steel sheet piling to be left in place will be measured by the square meter (square foot).

SECTION 206, AFTER LINE 244, INSERT AS FOLLOWS:

Steel sheet piling required for railroad protection will be paid for at the contract unit price per square meter (square foot) for sheet piling, steel, of the specified section modulus.

SECTION 206, AFTER LINE 327, INSERT AS FOLLOWS:

Sheet Piling, Steel, S = _____________	m2 (SFT)

section modulus

SECTION 206, AFTER LINE 334, INSERT AS FOLLOWS:

The costs of increased section modulus and cutting of steel sheet piling to the elevation specified shall be included in the cost of sheet piling, steel.