GORE AREA TREATMENT

The Standard Specifications are revised as follows:

SECTION 208, LINE 7, INSERT AS FOLLOWS:

with the elevations and cross sections shown on the plans or as directed. This work shall also consist of grading in gore areas and placement of HMA mixtures as shown on the plans.

SECTION 208, BEGIN LINE 57, DELETE AND INSERT AS FOLLOWS:

208.07 Method of Measurement. Grading of gore areas will be measured per each gore area graded. HMA mixtures required at gore areas will be measured by the megagram (ton). Finishing will not be measured for payment unless otherwise provided.

60		208.08 Basis of Payment. Grading in gore areas will be paid for at the contract unit price per each for grading of gore area. HMA mixtures required at gore areas will be paid for at the contract unit price per megagram (ton) for HMA for approaches.

Payment will be made under:

Pay Item	Metric Pay Unit Symbol (English Pay Unit Symbol)

HMA for Approaches	Mg(TON)

Grading in Gore Area	EACH

Finishing The costs of finishing shoulders, ditches, and slopes, all cut, fill including borrow, grading, and related earthwork for each gore area treatment will not be

paid for directly, the cost thereof shall be included in the costs of other pay items.

44444444444444444444444444444444444444444444444444444444444444444444444444