MAINTAINING TRAFFIC FOR MAINTENANCE OR RESURFACE WORK

The Standard Specifications are revised as follows:

SECTION 801, AFTER LINE 118, INSERT AS FOLLOWS:

Traffic shall be maintained for maintenance activities or for HMA resurface work as shown on the plans or as described herein. The Contractor shall have an extra set of construction signs and an extra flashing arrow sign on the project site so that the taper may be moved forward without suspending the operations and clearing the work area.  Additional traffic control devices shall be furnished for situations determined to be more complex, for protection in hazardous areas, and when traffic conditions warrant.

All nonfixed signs shall be removed at the completion of each day's operations. All lanes shall be open to normal traffic during hours other than daylight hours.  If a traffic lane is directed to remain closed during hours other than daylight hours, traffic shall be maintained on the remaining lanes as shown on the plans. A night watchman shall be provided to maintain drums, barricades, signs, or lights during such hours other than daylight hours.

All fixed signs shall remain in place until all temporary pavement markings have been removed. Work days will not be charged from the time of completion of other work until the markings have been removed.

44444444444444444444444444444444444444444444444444444444444444444444444444