###### National Pollutant Discharge Elimination System (NPDES)

Concentrated Animal Feeding Operation (CAFO) Individual Permit

Marketing or Distribution of Manure Information Sheet (Required Information)

*In addition to providing this form to each recipient, you should also keep a copy for your own records*

### Marketing Requirements

327 IAC 15-16-9(i) states in part, the owner or operator shall provide an information sheet to any person

that receives or purchases more than ten (10) cubic yards or four thousand (4,000) gallons of manure, litter, or process wastewater in a year from the CAFO unless the CAFO owner or operator takes responsibility for applying the manure, litter, or process wastewater.

327 IAC 15-16-9(i) states, the information sheet must contain, at a minimum, the following information:

1. The name and address of the CAFO providing the manure.

2. A statement indicating that it is unlawful to allow the manure to enter any waters of the state.

3. Information on the nutrient content of the manure, litter, and process wastewater, based on the most

current analysis.

4. The manure, litter, and process waste water application requirements of 327 IAC 15-16 (see below).

The owner/operator must maintain records of manure recipients and the quantities received.  The following information complies with the minimum standards imposed by 327 IAC 15-16-9.

### Marketing Information

Manure Recipient Name and Address:

CAFO Owner and Address:

Manure Nutrient Content:		Nitrogen		Phosphorous		Potassium (optional)

(use appropriate unit)

#/1000 gals.

#/cu.yd.

#/ton

Quantity Received:						Date Received:

APPLICATION RESTRICTIONS:

- It is unlawful to allow manure to enter any waters of the state.

- Manure must be applied in accordance with 327 IAC 15-16 (see below for guidelines).

327 IAC 15-16 Guidelines for Land Application of Manure

Required Acreage for Manure Application:

Manure, litter, and process wastewater should be applied to an adequate number of acres as calculated utilizing agronomic rates for current or planned crops regarding nitrogen and phosphorous demands, laboratory soil test results, the above supplied nutrient content of the received manure, and consideration of additional nitrogen and phosphorous inputs.

Required Setback Distances:

Except as otherwise provided, application of manure, litter, and process wastewater, must be in accordance with the setbacks in Table 1.  All setback distances must be measured from the edge of the feature to actual placement of manure, litter, or process wastewater on the land.

Table 1.  Setback Distances from Downgradient Surface Features (in feet)

Exceptions to setback distances in Table 1:

- The property line setback distances may be waived in writing by the owner of adjoining property.

- If a properly designed and maintained filter strip of at least fifty (50) feet in width is located between the application site and surface waters of the state, any known private well, the surface opening or lowest point of any sinkhole, or any drainage inlet, including water and sediment control basins then the setback is the width of the filter strip.

- The setback is ten (10) feet if a gradient barrier is located between the application site and surface waters of the state, any known well, the surface opening or lowest point of any sinkhole, or any drainage inlet, including water and sediment control basins.

Prohibitions:

1.	Surface land application of manure, litter, and process wastewater is prohibited to snow covered or frozen ground, unless in accordance with the Fertilizer Material Use, Distribution, and Record Keeping Rule (June 2012) developed by the  Office of Indiana State Chemist.

2.	Manure, litter, and process wastewater shall not be applied to saturated ground.

3.	Manure, litter, or process wastewater must not be applied to the land from manure application equipment operating on a public road.

4.	Manure, litter, or process wastewater must not be applied by spray irrigation to any land that has less than twenty (20) inches of soil above the bedrock.

5.	Spray irrigation is prohibited in a flood plain unless the setback from surface water is increased to 200 feet, spraying is only done during months that the current soil survey book indicates have a low potential for flooding, and there is  no expectation of flooding based on available weather forecast information and rainfall conditions upstream.

6.	Spray irrigation of liquid manure or process wastewater must be conducted to prevent equipment leaks and excessive application.  Application is deemed excessive when the application rate exceeds the infiltration rate of the soil where the application is occurring, expressed in inches per hour.

7.	Spray irrigation must be conducted under the constant supervision or a person or with devices to detect pressure loss due to leaks and devices to shut down the system if leaks are detected.

8.	Manure, litter, and process wastewater must not be applied to highly erodible land unless the land is patureland, the land is planted in a cover crop that reduces or controls erosion, or the manure, litter, or process wastewater is applied in accordance with a soil conservation practice plan.

9.	When planning land application, take into account the weather forecast and the likelihood of precipitation events for twenty-four (24) hour period prior to and after the application and site soil conditions to assure that manure, litter, or process wastewater are not applied prior to a rain event that, when combined with soil conditions, would likely result  in runoff.

10.	Land application sites must be inspected to identify any field tile outlets under or immediately bordering the land application site.  Visual monitoring of identified field tile outlets must occur during and immediately following land application of manure, litter, or process wastewater.  If there is evidence of process wastewater discharging from the field tile outlet, the land application must cease immediately and the flow stopped or captured.  Any flow that is captured must be land applied.