ATTACHMENT I

# INDIANA DEPARTMENT OF TRANSPORTATION (INDOT),

# OTHER STATE AGENCIES (OSAs)

# AND LOCAL GOVERNMENTAL ENTITIES

# 2026/2027 ROAD SALT SPECIFICATIONS

FOR UNTREATED SODIUM CHLORIDE & SODIUM CHLORIDE TREATED

WITH MAGNESIUM CHLORIDE

These specifications, terms, and conditions apply to salt for INDOT, OSA, and Local Governmental Entity locations identified within this solicitation.

# 1.0 SCOPE OF WORK

The work to be performed consists of furnishing and delivering Sodium Chloride and Sodium Chloride treated with magnesium chloride to various locations throughout Indiana for use in winter maintenance operations. Sodium Chloride and Sodium Chloride treated with Liquid Magnesium Chloride will be in accordance with the 2026 Indiana Department of Transportation Standard Specifications (Sections 913.03) and the Supplemental Specifications at bid due date and time, and all applicable rules and regulations at the federal, state and local levels.

# 2.0 SPECIFICATIONS

All material furnished shall be (1) uniform in appearance, free flowing, free of visual evidence of foreign matter including but not limited to dirt, stone, trash, or any other material; (2) chemically treated to prevent caking, and shall be free of foreign matter, lumps, and free water.

Sodium Chloride

Sodium Chloride shall be rock salt or solar salt conforming to the requirements of AASHTO M 143, Type I Grade I, with a moisture content not exceeding two (2) percent.

Sodium Chloride treated with Liquid Magnesium Chloride

Sodium Chloride treated with Liquid Magnesium Chloride shall be rock salt or solar salt conforming to the requirements of AASHTO M 143, Type I Grade I, treated with Liquid Magnesium Chloride with a moisture content not exceeding five and three tenths (5.3) percent.

The finished product shall not be less than 91.3% sodium chloride determined as follows: Apparent total % sodium chloride shall be determined in accordance with ASTM-D-632. Magnesium and calcium content shall be determined in accordance with ASTM E-534 and computed as % magnesium chloride and % calcium chloride respectively. % sodium chloride shall then be computed as follows:

% sodium chloride = % apparent sodium chloride – (% magnesium chloride + % calcium chloride)

The sampling shall be in accordance with Indiana Testing Method (ITM) 810. Sampling frequency shall be as set out in the INDOT manual entitled, "Manual for Frequency of Sampling and Testing and Basis for Use of Materials, revised June 2025". INDOT reserves the right to sample more frequently at their discretion. The rapid test method referenced in AASHTO M 143, Annex A1 shall be used for both routine acceptances testing and for resolution of disputes concerning chemical composition. Material not complying with these requirements shall be paid for at a reduced price as set out under "Deductions" in the Special Provisions (See Section 8.0). For further clarification, see Section 8.1 Testing & Deduction Clarifications.

# 3.0 	INSPECTION

All Sodium Chloride and Sodium Chloride treated with Magnesium Chloride delivered will be visually inspected at time of delivery and samples taken for laboratory analysis of gradation, purity, and moisture content in accordance with Section 2.0.

# 4.0	REJECTION

Any material delivered which contains lumps, foreign matter or free water shall be rejected. In the event the material has been loaded or dumped prior to rejection it shall be immediately reloaded or removed by the Contractor within 48 hours of notification of rejection. Failure to remove the material will result in liquidated damages in the amount of $10.00 per ton for each calendar day the salt remains on INDOT/OSA/Local Governmental Entity property.

# 5.0	DELIVERY

Deliveries shall be made during regular working hours when possible and will be accepted at other times only when prior arrangements have been approved by the District Highway Maintenance Director, designee, or the District personnel responsible for coordinating salt deliveries (INDOT only). For OSAs and Local Governmental Entities, arrangements will need to be approved by the designated contact responsible for coordinating salt deliveries. No payments from INDOT will be made for any load for which a delivery ticket, signed by an INDOT representative, cannot be produced. Delivery tickets for all deliveries shall indicate gross, tare, and net weights, order number, locations of stockpile from which shipment is made, and point of delivery. Weights will be checked at random for accuracy of the delivery ticket weights. INDOT, OSAs, and Local Governmental Entities reserve the right to require any truck to go to the nearest available certified scales to check weights at no additional cost to INDOT, OSAs, and Local Governmental Entities.

A.	Routine Deliveries Beyond Early Delivery Final Date

The minimum order quantity to qualify for delivery is one (1) truckload (22-25 tons).   Orders less than one (1) truckload (22-25 tons) will require arrangements with vendor to be picked-up from vendor's depot.

Contractors shall make delivery in trucks with solid or waterproof tarps to stockpile locations within any District for which it has received Award of Contract. Deliveries must be pre-scheduled with district designated personnel a minimum of 1 business day prior to the anticipated delivery date. Scheduling must include anticipated quantities, locations and timeframe for delivery.

Delivery must be 100% completed within ten (10) business days after placement of order. For each business day that delivery extends beyond this 10-day limit, INDOT, IDOA, or the OSA or Local Government Entity placing the order may assess liquidated damages in accordance with Section 53 of the Contract. When the Contractor contacts the Ordering Entity to confirm delivery (as required above), should the Ordering Entity no longer need the salt originally ordered, the Contractor will provide the liquidated damages amount in the form of a check to the Ordering Entity. In this instance, the amount shall be calculated beginning on the tenth business day and spanning to the date of order cancellation. Should there be any discrepancies, INDOT or IDOA shall evaluate the situation and provide guidance.

Orders shall be placed by telephone, email, or Contractor-provided online ordering portal to the office specified by the Contractor. The Contractor should provide a single telephone number, a single email address, and a single online ordering portal web address for all orders made for each purchase order. The Contractor shall provide order confirmation via e-mail or fax, so that each Ordering Entity can provide verification for each order placed.

B.	Early Delivery Period

The Contractor shall furnish and deliver the Early Storage Requirements at the locations listed on the attached sheets (Bid List.xls, “Early Storage Requirements” spreadsheet) at all salt storage buildings or outside storage areas as follows:

  - La Porte and Fort Wayne INDOT Districts: Initial 50% of order delivered on or before October 15, 2026, and remaining 50% delivered on or before November 15, 2026.

  - Crawfordsville, Greenfield, Seymour, and Vincennes INDOT Districts: Initial 50% of order delivered on or before November 1, 2026, and remaining 50% delivered on or before December 1, 2026.

For each business day that early storage requirements for each bidding unit are not complete after the time specified, INDOT reserves the right to assess liquidated damages at two percent (2%) per day not to exceed a maximum of 20% of the order. INDOT is committed to purchasing 100% of the Early Fill amounts listed by INDOT district (as listed in the INDOT Early Fill Requirement spreadsheet), not sub-district.

# 6.0	BASIS OF PAYMENT

INDOT, OSAs, and Local Governmental Entities shall pay the negotiated Contract per-ton-price for the type of salt (as specified in Bid List.xls), furnished and delivered to the various locations as designated. There shall be no other charges. Pricing submitted by respondents will be the price for purchases from 0% to the maximum of the commitment range of 120%. Any additional tons needed beyond the maximum commitment range (120%) will be by mutual agreement of the parties, including the price thereof.

The quantities shown by INDOT sub-district are estimates only and may be adjusted at the option of INDOT. INDOT commits to purchase 100% of total tonnage designated as Early Fill by district, and at least 80% of total tonnage designated as Seasonal Fill by district. Each OSA and Local Governmental Entity commits to purchase 80% of each entity’s requested tonnage. The quantities in the Att D Cost Proposal are 100%.

- 80-120% Commitment Range - All INDOT Districts (Crawfordsville, Fort Wayne, Greenfield, LaPorte, Seymour, and Vincennes)

  - “Delivered” is price of salt per ton delivered to the INDOT, OSA, or Local Governmental Entity location

  - “Delivered + Loaded” is price of salt per ton to deliver to the INDOT, OSA, or Local Governmental Entity location and load the salt via a conveyor (see section 9.0 for loading requirements)

  - “Pick-Up” is the price of the salt per ton if picked up from the depot by the Ordering Entity

# 7.0	INVOICING

INDOT, OSAs, and Local Governmental Entities require the Contractor to invoice INDOT, the OSA, or the Local Governmental Entity for each Subdistrict location (or OSA or Local Governmental Entity facility) where business has been transacted. Invoices shall itemize the daily activity for that Subdistrict/location.

In accordance with Section 5.0, DELIVERY and Section 8.0, DEDUCTIONS of the Specifications & Special Provisions, INDOT, OSAs, and Local Governmental Entities will notify the Contractor(s) in writing for each Unit or Subdistrict location (or OSA or Local Governmental Entity facility) when liquidated damages for late deliveries or deductions for non-compliance with specifications are being assessed. The Contractor(s) shall submit a credit memorandum to INDOT, the OSA, or the Local Governmental Entity for the assessed liquidated damages or deductions within fifteen (15) calendar days from the date of written notification from INDOT, the OSA, or the Local Governmental Entity.

Payment will be made following necessary testing and evaluation as described in the contract terms (See section 8.0 Deductions and section 8.1 Testing & Deduction Clarifications). INDOT, OSAs, and Local Governmental Entities shall complete necessary testing and evaluation within ten (10) business days after an order has been fully received; if the department fails to test and evaluate within this timeframe, liquidated damages, or deductions for non-compliance with specifications shall not be assessed.

# 8.0	DEDUCTIONS

After testing sodium chloride, deductions will be made for non-compliance with specifications on the following basis:

- CHEMICAL COMPOSITION

Sodium Chloride

Results of the purity test shall be rounded up to the nearest whole percentage point. (.5 rounded up). A deduction of $1.00 (one dollar) per ton will be made for each percentage point from 94 percent through 90 percent and $2.00 (two dollars) per ton for each percentage point from 89 percent through 85 percent. Material with purity less than 84.5 percent will be paid for as snow and ice abrasives at a rate of $4.00 (four dollars) per ton.

Sodium Chloride treated with Liquid Magnesium Chloride

Results of the purity test shall be rounded up to the nearest whole percentage point. (.5 rounded up). A deduction of $1.00 (one dollar) per ton will be made for each percentage point from 91 percent through 87 percent and $2.00 (two dollars) per ton for each percentage point from 86 percent through 82 percent. Material with purity less than 81.5 percent will be paid for as snow and ice abrasives at a rate of $4.00 (four dollars) per ton.

| Deduction | Sodium Chloride | Sodium Chloride treated with Liquid Magnesium Chloride |
| --- | --- | --- |
| $1.00 per Ton | 94 – 90 % | 91 – 87 % |
| $2.00 per Ton | 89 – 85 % | 86 – 82 % |
| Paid at $4.00 per Ton | Less than 84.5 % | Less than 81.5 % |

- MOISTURE

Sodium Chloride

If the moisture content exceeds two (2) percent, the weight to be paid for will be the gross weight of the Sodium Chloride minus twice the weight of the excess moisture computed as follows:

Weight to be paid for = G x (104-2(m)) /100

G=Gross weight of material (wet).

M=Percent of moisture to the nearest 0.5 percent based on oven dry weight.

Sodium Chloride treated with Liquid Magnesium Chloride

If the moisture content exceeds five and three tenths (5.3) percent, the weight to be paid for will be the gross weight of the Sodium Chloride minus twice the weight of the excess moisture computed as follows:

M=Percent of moisture to the nearest 0.5 percent based on oven dry weight.

- GRADATION

| Sieve Sizes | Percent Passing |
| --- | --- |
| ½ inch (12.5mm) | 95 - 100 |
| 3/8 inch (9.5 mm) | 92 - 100 |
| No. 4 (4.75 mm) | 15 - 95 |
| No. 8 (2.36 mm) | 10 - 65 |
| No. 30 (0.60 mm) | 0 - 20 |

When test results for the Sodium Chloride furnished exceed the gradation requirements above, adjustment points will be assessed as follows:

ADJUSTMENT POINTS FOR GRADATION

Sieve Size

| Adjustment Points | ½ inch | 3/8 inch | No. 4 | No. 8 | No. 30 |
| --- | --- | --- | --- | --- | --- |
|  | 12.5 mm | 9.5 mm | 4.75 mm | 2.36 mm | 0.60 mm |
| For each 1.0% up to 3.0% |  |  |  |  |  |
| Out of Tolerance | 1.0 | 1.0 | 1.0 | 1.0 | 2.0 |
| For each 1.0% > 3.0% |  |  |  |  |  |
| Out of Tolerance | 1.0 | 1.0 | 1.0 | 1.0 | 3 |

Gradation adjustment points for the quantity represented shall be the sum of points calculated for up to 3% out of tolerance and the points calculated for greater than 3% out of tolerance.

The minimum required number of tests will be as set out in “INDOT Manual for Frequency of Sampling and Testing and Basis for Use of Material,” revised June 2025. Samples will be taken by or under the supervision of a representative of the department. All materials being used are subject to inspection, test, or rejection at any time.

Where INDOT, the OSA, or the Local Governmental Entity determines that a sample does not meet specifications in chemical composition, moisture and gradation, the following shall be the method of determining the final price per ton:

- First, tons eligible for payment shall be calculated as noted in Section B. Moisture.

- Second, a deduction, as specified above, will be made for gradation failure.

- Finally, the deduction will be determined for chemical composition, unless the chemical composition falls below

84.5 percent at which time the entire amount will be paid for as snow and ice abrasives as noted in Section A. Chemical Composition.

# 8.1	TESTING AND DEDUCTION CLARIFICATIONS

    - For the purposes of testing and deductions, an order shall be considered a request for salt delivery to a single physical location.

    - The INDOT Salt Testing Calculator shall be used to settle disputes regarding amounts of liquidated damages or deductions between the Contractor and the Ordering Entity.

    - Testing is to be completed on every order regardless of tonnage from each depot.

      - If the order is larger than 2000 tons, then it will be required to be tested every 2000 tons (E.g., a 4200-ton order would need 3 tests).

      - If the order is less than 2000 tons, then it is still required to be tested (E.g., an 800-ton order still needs a test completed).

    - Penalties will be based on orders.

      - If the order is larger than 2000 tons, then INDOT, OSAs, and Local Governmental Entities can penalize each test, per depot, up to 2000 tons (E.g., a 3800-ton order fails one test but not the other penalizes 2000 tons).

      - If the order is less than 2000 tons, INDOT, OSAs, and Local Governmental Entities can penalize per order up to the amount ordered (E.g., a 1400-ton order fails test then penalize 1400 tons).

# 9.0	REQUIREMENTS FOR DELIVERY

The following special provisions apply to all INDOT, OSA, and Local Governmental Entity salt storage sites, except where noted on the Special Provision pages.

- Delivered and loaded shall mean placement of salt in the departments designated storage buildings, with equipment and labor furnished by the Contractor or the Contractor’s appointed hauler. Loading shall be directly from the truck to the storage building, by way of loading equipment. Salt shall not be placed on the ground outside a building prior to loading in the building. If placing salt on the ground outside of a building prior to loading is necessary because of operational or safety concerns, this action must be approved by the District Highway Maintenance Director, designee, or the district personnel responsible for coordinating salt deliveries (INDOT only). For OSAs and Local Governmental Entities, approval must be given by the designated contact responsible for coordinating salt deliveries.

- Loading equipment shall be provided by the Contractor or the Contractor’s appointed hauler, which shall be capable of fully loading INDOT, OSA, and Local Governmental Entity storage buildings. The Contractor may review specific sites to determine equipment required. A pre-planned time for such visits is required. Loading equipment should include conveyors or other equipment as approved by INDOT, OSAs, and Local Governmental Entities. The Contractor shall use commercially reasonable best efforts to make additional conveyors (more than one) available upon request.

- Failure to load salt in the INDOT, OSA, or Local Governmental Entity’s salt storage building will result in a deduction, as liquidated damages, from the price bid for salt delivered and loaded. The amount shall be $1.00 per ton plus the price differential charged for the loaded price.

- Delivered No Deduction; shall mean salt that is delivered but not loaded in the designated storage buildings. This delivery method must be pre-approved by the INDOT District Highway Management Director, OSA contact, or Local Governmental Entity contact and marked on the delivery ticket as such. Deliveries without prior authorization and the salt not loaded in the building will result in liquidated damage as listed above.

- Delivery tickets must be marked by the Ordering Entity as:

“Delivered and Loaded” to indicate full payment for delivery and loaded as bid. (Salt properly placed in the building.)

“Delivered – Not Loaded, Deduction Required” to indicate material delivered but not loaded resulting in the deduction for liquidated damages above. (This represents dumped without authorization.)

“Delivered - No Deductions” to indicate that material was delivered as requested.

The Contractor is required to provide delivery ticket in the format that contains the above terms to facilitate faster actions.

- The Contractor will be responsible for any damage to the salt shed resulting from improper piling of salt. Further, the Contractor may be required to move any material improperly stacked.

- Conveyor loading only is acceptable at all Districts. Slinger loading is not acceptable. INDOT, OSAs, and Local Governmental Entities reserve the right to reject Contractor loading if the price or quantity being delivered is unacceptable.

# 10.	DEFINITIONS AND TERMS

For the purposes of this contract, calendar day and business day shall be defined as:

Calendar Day – Every day shown on the calendar.

Business Day – A calendar day, exclusive of Saturdays, Sundays, and State recognized legal holidays.

REFERENCED

INDIANA DEPARTMENT OF TRANSPORTATION STANDARD SPECIFICATIONS 2026

(With Supplemental Specifications in effect at time of letting)

http://www.in.gov/dot/div/contracts/standards/book/index.html

SECTION 913 - MISCELLANEOUS

(913.03 Sodium Chloride. Sodium chloride shall be in accordance with AASHTO M 143). Rock salt shall be used for de- icing purposes. Either rock salt or evaporated salt may be used for stabilization.

MANUAL FOR FREQUENCY OF SAMPLING AND TESTING AND BASIS FOR USE OF MATERIAL, REVISED, JUNE 2025.

https://www.in.gov/indot/doing-business-with-indot/files/frequency-manual.pdf

INDIANA TEST METHOD OR PROCEDURE

ITM 810-15 Deicing Material, Dated June 16, 2015

https://www.in.gov/indot/doing-business-with-indot/files/810_testing.pdf