7’ x 22' (16' +6') Heavy Duty 14K Low Profile Gooseneck Tilt Deck Equipment Hauler Flatbed Trailer.

Heavy Duty 6" Channel Main Frame

3 x 3/16" Channel Cross Members

16'' Crossmembers on center

14,000 lb G.V.W.R.

(2) 7,000 lb Nevr-R-Adjust Torsion All Wheel Electric Brake Axles

All Wheel Electric Brakes

Emergency Break-A-Way Kit

7-Way Round Electrical Plug

3" x 10" Cylinder with 1.5" Shaft

2 5/16" Adjustable Heavy Duty Gooseneck Coupler

Front Locking Storage Box

Treated Wood Deck

Heavy Duty Diamond Plate Steel Removable Fenders

Heavy Duty Safety Chains - w/ Hooks

2 = 10,000 lb Drop Leg Jacks

Rub Rails, Headache Bar (Front Stop Rail)

20" Deck Height

11 Degree Loading Angle

Knife Edge Tail

(4) 3" D-Rings and Stake Pockets (For Tie Downs)

Tires: ST235-80R-16 LRE 10 Ply Radial Tires

Wheels: 16" Mod Wheels

Powder Coated Paint & One Coat Cure Primer

Lifetime Recessed L.E.D. Lighting

All Lighting D.O.T. Approved

D.O.T. Tape

Bed Width: 83" Between Fenders

Deck Length: 22' (6' Stationary Deck + 16' Gravity Tilt Deck w/ Knife Edge Tail)