Extended Inspection Interval Request – NSTM Inspection

| Bridge Number (B.IE.01) |  |
| --- | --- |
| Bridge Name (B.IE.02) |  |

Responsible Bridge NSTM Inspection Team Leader (ATL-NSTM) shall review and check the boxes for each extended interval criteria, attesting that each criterion has been satisfied.

| ☐ | The Year Built (SNBI B.W.01) is coded 1990 or after. |
| --- | --- |
| ☐ | Design plans and shop drawings are available and show that all NSTMs were fabricated in accordance with a fracture control plan, and are all clearly identified on final design plans and shop drawings and are also detailed in the NSTM Inspection POA. |
| ☐ | The bridge containing NSTMs has not been either newly constructed or recently undergone rehabilitation construction. |
| ☐ | All NSTMs that are part of the same bridge asset are rated 6 (Satisfactory) or better condition and the NSTM Inspection Condition (SNBI B.C.14) is rated 6 (Satisfactory) or greater. |
| ☐ | All NSTMs that are part of the same bridge asset have no fatigue details with finite life (no E or E’ details present) directly on any individual NSTM. |
| ☐ | All NSTMs that are part of the same bridge asset have no history of fatigue cracks. |
| ☐ | All NSTMs that are part of the same bridge asset do not have either pin and hanger or pin and hinge assemblies. |
| ☐ | The Inventory Load Rating Factor (SNBI B.LR.05) for the bridge asset with NSTMs is coded 1.0 or greater for the standard AASHTO HS-20 or HL-93 loadings. |
| ☐ | The Routine Permit Loads (SNBI B.LR.08) for the bridge asset with NSTMs is coded either A or N reporting that routine permit loads are either not restricted, not carried, or not issued. |
| ☐ | The reported Bridge Width Curb-to-Curb (SNBI B.G.06) is greater than 18.0 feet. |
| ☐ | The reported Annual Average Daily Truck Traffic (SNBI B.H.10) for the bridge asset with NSTMs is less than 14,500. |
| ☐ | The structure is not coded as a complex bridge. |
| ☐ | The NSTM Inspection has been approved in writing by the INDOT Bridge Inspection SPM for the extended interval, and a copy of the extended interval authorization is in the bridge asset file. |

By typing or signing your name below, you are certifying and validating that all the criteria requirements for NSTM extended inspection intervals on this form are truthful and accurate to the best of your knowledge.

Approved NSTM Team Leader (ATL-NSTM)					Date

Bridge Inspection State Program Manager (SPM) Approval

INDOT Bridge Inspection SPM							Date