| Structure Type | Infiltration Swale |  |  | Number |  |
| --- | --- | --- | --- | --- | --- |
| Design Criteria |  |  |  | Location | Coordinates, Driving Directions |
| This swale was designed to remove Total Suspended Solids (TSS) from stormwater runoff and was designed to be dry between rainfall events. Stormwater should infiltrate into the underlying soil. |  |  |  |  |  |
| Inspection Cycle |  |  |  |  |  |
| Twice per year during first year after construction, then one time per year. Also inspect after major storm events (6 inches of rainfall). |  |  |  |  |  |
| Inspection Criteria |  |  |  |  |  |
| - Vegetation – cover should be approximately 90% - Erosion and scour - Trash and debris buildup - Excessive ponding – stagnated water - Inflow and outflow points and/or structures are not blocked or damaged - Sediment buildup – should be ≤ 25% of original design volume |  |  |  |  |  |
| Typical Corrective Actions |  |  |  |  |  |
| - Vegetation – re-establish vegetation as needed so that cover is approximately 90% - Erosion and scour – re-grade as needed, install erosion protection if required - Trash and debris buildup – remove trash and debris as needed - Excessive ponding – regrade as needed to drain excessive ponded or stagnated water, remove top 3 inches of soil if swale becomes clogged - Inflow and outflow points and/or structures – repair structures and remove debris or blockage as needed - Sediment buildup – should be ≤ 25% of original design volume – remove sediment as needed |  |  |  |  |  |
| Maintenance Recommendations |  |  |  |  |  |
| Last Inspected |  | Current Inspection |  |  |  |

| Structure Type |  |  |  | Number |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Plans and Plan Cross Section(s) |  |  |  |  |  |  |
| INSPECTED BY |  | APPROVED BY |  |  |  |  |
| Printed Name/Title |  |  |  |  |  |  |
|  |  |  | Printed Name/Title |  |  |  |