| Structure Type | Turf Grass Filter Strip |  |  | Number |  |
| --- | --- | --- | --- | --- | --- |
| Design Criteria |  |  |  | Location | Coordinates, Driving Directions |
| This vegetated turf grass filter strip was designed to remove Total Suspended Solids (TSS) from stormwater runoff. Turf grass height should be at least 6 inches for adequate TSS removal. Stormwater should travel as sheet flow across the filter strip. |  |  |  |  |  |
| Inspection Cycle |  |  |  |  |  |
| Twice per year during first year after construction, then one time per year. |  |  |  |  |  |
| Inspection Criteria |  |  |  |  |  |
| - Vegetation – cover should be approximately 90% - Scour or rill and gully erosion - Trash and debris buildup - Ponding or concentrated flow paths |  |  |  |  |  |
| Typical Corrective Actions |  |  |  |  |  |
| - Vegetation – re-establish turf grass as needed so that cover is approximately 90% - Erosion and scour – re-grade as needed, install erosion protection if required - Trash and debris buildup – remove trash and debris as needed - Ponding or concentrated flow – regrade as needed so that stormwater travels as sheet flow across the filter strip - Inflow and outflow points and/or structures – repair structures and remove debris or blockage as needed |  |  |  |  |  |
| Maintenance Recommendations |  |  |  |  |  |
| Last Inspected |  | Current Inspection |  |  |  |

| Structure Type |  |  |  | Number |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Plans and Plan Cross Section(s) |  |  |  |  |  |  |
| INSPECTED BY |  | APPROVED BY |  |  |  |  |
| Printed Name/Title |  |  |  |  |  |  |
|  |  |  | Printed Name/Title |  |  |  |