| Structure Type | Wet Retention Pond |  |  | Number |  |
| --- | --- | --- | --- | --- | --- |
| Design Criteria |  |  |  | Location | Coordinates, Driving Directions |
| This wet retention pond was designed to remove Total Suspended Solids (TSS) from stormwater runoff but also serves as a peak flow mitigation measure. This pond should have a permanent pool of water. |  |  |  |  |  |
| Inspection Cycle |  |  |  |  |  |
| Twice per year and after major storm events (6 inches of rainfall). |  |  |  |  |  |
| Inspection Criteria |  |  |  |  |  |
| - Vegetation – cover should be approximately 90% in areas without permanent pool - Erosion and scour - Trash and debris buildup - Emergency overflow structure or embankment – structural integrity – no trees or large vegetation on earth embankments - Inflow and outflow points and/or structures are not blocked or damaged - Sediment buildup - Algal growth – excessive algal growth or cyanobacteria (bright green algae often with a film on the surface) |  |  |  |  |  |
| Typical Corrective Actions |  |  |  |  |  |
| - Vegetation – re-establish as needed so that cover is approximately 90% in areas without permanent pool - Erosion and scour – re-grade as needed, install erosion protection if required - Trash and debris buildup – remove trash and debris as needed - Emergency overflow structure or embankment – repair structures and remove large vegetation or trees from embankments as needed - Inflow and outflow points and/or structures – repair structures and remove debris or blockage as needed - Sediment buildup – should be ≤ 25% of original design volume – remove sediment as needed - Algal growth – apply aquatic herbicide to treat excessive algal growth or cyanobacteria – must be Category 5 certified |  |  |  |  |  |
| Maintenance Recommendations |  |  |  |  |  |
| Last Inspected |  | Current Inspection |  |  |  |

| Structure Type |  |  |  | Number |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Plans and Plan Cross Section(s) |  |  |  |  |  |  |
| INSPECTED BY |  | APPROVED BY |  |  |  |  |
| Printed Name/Title |  |  |  |  |  |  |
|  |  |  | Printed Name/Title |  |  |  |