DES. # 1800163

LEGAL NOTICE OF PLANNED IMPROVEMENT

The Indiana Department of Transportation (INDOT) and the Federal Highway Administration are developing plans for a proposed slide correction project on State Road (SR) 145, approximately 1.69 miles south of SR 62 junction in Perry County, Indiana.

The purpose of this project is to restore this section of SR 145 which was damaged by landslide activity, and reinforce the failed slope, which will reduce the potential for future slide activity, resulting in improved traffic mobility and safety for the traveling public.  The need for the project is due to pavement cracking and sinking caused by the lateral slide of the embankment, threatening the structural integrity of the roadway.

The project proposes to correct the slide by excavating and replacing the failed soil with compacted soil and repaving the section of SR 145 that was damaged by the slide. An existing 18-inch pipe at the north end of the slide limits will be extended by about 55 feet. The project will include approximately 250 feet of stream realignment to Unnamed Tributary (UNT) 1 to Anderson River. Furthermore, the existing embankment of UNT1 to Anderson River will be stabilized using revetment riprap over geotextile fabric.

The proposed construction of this project will require 0.81 acre of new permanent right-of-way. No temporary right-of-way is needed for this project. No relocations or displacements will occur as a result of this project.

The maintenance of traffic (MOT) plan proposes a road closure with an official INDOT detour. An official state detour utilizing I-64, SR 37, and SR 145 will be used during construction.  Local roads may be used by local traffic.  Access to all properties will be maintained throughout construction.  School corporations and emergency services will be notified of closures prior to construction.  The proposed start of construction is spring of 2023.

The project will impact a total of 275 linear feet of UNT1 to Anderson River. A total of 0.30 acre of tree removal is anticipated. The cost associated with this project is approximately $663,000.00, which includes right-of-way and construction monies with both federal and state funds anticipated to be used.  The Federal Highway Administration (FHWA) and INDOT have agreed this project falls within the guidelines of a Categorical Exclusion (CE) Level 2 environmental document.  Preliminary design plans along with the draft CE are available for review at the following locations:

- Ferdinand Branch Library – 112 E 16th St, Ferdinand, IN 47532

- INDOT Vincennes District website – https://www.in.gov/indot/2707.htm

All interested persons may request a public hearing be held and/or express their concerns by submitting comments to the attention of Payton Parke at (812) 759-4119 or at pparke@lochgroup.com on or before June 18, 2021. Persons with limited internet access may request project information to be mailed, please contact Payton Parke Lochmueller Group, Inc.

In accordance with the Americans with Disabilities Act (ADA), persons and/or groups requiring project information be made available in alternative formats are encouraged to contact the INDOT Office of Public Involvement for the arrangement and coordination of services.  Please contact Payton Parke at (812) 759-4119 or at pparke@lochgroup.com. In accordance with Title VI of the Civil Rights Act of 1964, persons and/or groups requiring project information be made available in another language are encouraged to contact Payton Parke, Lochmueller Group, Inc.

This notice is published in compliance with: 1) Code of Federal Regulations, Title 23, Section 771 (CFR 771.111(h)(1) stating, “Each State must have procedures approved by the FHWA to carry out a public involvement/public hearing program.”; 2) 23 CFR 450.210(a)(1)(ix) stating, “Provide for the periodic review of the effectiveness of the public involvement process to ensure that the process provides full and open access to all interested parties and revise the process, as appropriate.”; and 3) The INDOT Public Involvement Policies and Procedures approved by the Federal Highway Administration on August 16, 2012.