TITLE 170 INDIANA UTILITY REGULATORY COMMISSION

170 IAC 5-5

Draft Proposed Rule

LSA Document #22-___

IURC RM #22-03

DIGEST

Amends 170 IAC 5-5-1, 170 IAC 5-5-2, 170 IAC 5-5-3, 170 IAC 5-5-4 to make various updates to definitions, add definitions for re-mark, job extension, and additional notice tickets, to modify the procedural timeline for resolution of excavation damage cases, to require account information on the IURC website, and to require a publicly available penalty schedule from the UPPAC.

Adds 170 IAC 5-5-1.1, 170 IAC 5-5-1.2, 170 IAC 5-5-1.3, 170 IAC 5-5-2.1, 170 IAC 5-5-2.2, and 170 IAC 5-5-2.3 to define two full working days in which an operator shall provide location information, to specify the expiration date and time of 811 tickets, to require markings indicating the size and type of underground pipelines and infrastructure, to define compliance methods for notifying excavators under IC 8-1-26-18(k), to define the clearance for round facilities, and to require positive response to excavators.

Effective 30 days after filing with the Publisher.

## SECTION 1. 170 IAC 5-5-1 IS AMENDED TO READ AS FOLLOWS:

## 170 IAC 5-5-1 Definitions

Authority: IC 8-1-26-26, IC 8-1-26-16

Affected: IC 8-1-26

Sec. 1. (a) The definitions in IC 8-1-26, where applicable, and this section apply throughout this rule.

(b) “811 ticket” means a notice of excavation or demolition as described by IC 8-1-26-16.

(c) “Account” has the same meaning as set forth in IC 8-1-26-1.3.

(d) “Advisory committee” means the underground plant protection advisory committee has the same meaning as set forth in IC 8-1-26-1.5.

(e) “Association” has the same meaning as set forth in IC 8-1-26-3.

(b)(f) “Commission” means the Indiana utility regulatory commission.

(g) “Damage” has the same meaning as set forth in IC 8-1-26-4.

(h) “Demolish” has the same meaning as set forth in IC 8-1-26-5.

(i) “Excavate” has the same meaning as set forth in IC 8-1-26-6.

(j) “Pipeline facilities” has the same meaning as set forth in IC 8-1-26-11.2.

(k) “Other materials” as referenced by IC 8-1-26-6 includes, but is not limited to, the following:

(1)	Shrubs, trees, timber, and similar brush.

(2)	Posts and fencing.

(3)	Poles.

(4)	Stakes.

(l) “Operator” has the same meaning as set forth in IC 8-1-26-10.

(c)(m) “Pipeline safety division” means the pipeline safety division of the commission. or the pipeline safety division’s publicly noticed consultant.

(n) “Job Extension ticket” means an 811 ticket that references a previous 811 ticket with an identical scope and is treated as an 811 ticket in all respects.

(o) “Re-mark ticket” means an 811 ticket in which an excavator affirms that markings from a previous 811 ticket have become illegible, as required under IC 8-1-26-20(a)(3)(b). The re-mark ticket makes the previous 811 ticket invalid as of the notice to the association.

(p) “Additional notice” means a request for the association to notify operators that the markings from a previous 811 ticket:

(1)	were not provided, or

(2)	were provided but there is visual evidence of an unmarked underground facility in the area of the excavation or demolition.

An additional notice does not modify the expiration date of the previous 811 ticket.

(q) “Training” for purposes of IC 8-1-26-23(h)(2) means training provided by the commission, the pipeline safety division, or a person or entity approved by the commission.

(r) “Working day” has the same meaning as set forth in IC 8-1-26-12.

(Indiana Utility Regulatory Commission; 170 IAC 5-5-1; filed May 25, 2011, 12:57 p.m.: 20110622-IR-170100184FRA; readopted filed Apr 11, 2017, 9:52 a.m.: 20170510-IR-170170124RFA)

## SECTION 2. 170 IAC 5-5-1.1 IS ADDED AS FOLLOWS:

## 170 IAC 5-5-1.1 Deadline to Supply Marking Information

Authority: IC 8-1-26-26

Affected: IC 8-1-26-16, IC 8-1-26-18, IC 8-1-26-21

Sec. 1.1. After an 811 ticket is submitted, an operator shall supply the information required by IC 8-1-26-18 not later than 7:00 a.m., eastern standard time or eastern daylight savings time, as applicable in Greenwood, Indiana, on the next working day after the elapse of two full working day periods from 7:00 a.m. to 6:00 p.m.

(Indiana Utility Regulatory Commission; 170 IAC 5-5-1.1)

## SECTION 3. 170 IAC 5-5-1.2 IS ADDED AS FOLLOWS:

## 170 IAC 5-5-1.2 Location and description of underground facilities

Authority: IC 8-1-26-26

Affected: IC 8-1-26-16, IC 8-1-26-18, IC 8-1-26-21

Sec. 1.2. An operator required to supply the location and description of the underground facilities under IC 8-1-26-18 shall, as part of the description, include the following:

(1) A written description on the ground near the underground facility or in another manner that ensures the party performing the excavation or demolition receives the description.

(2) For pipeline facilities that are service lines,

an indication the underground pipeline facility is a service line.

(3) For pipeline facilities two (2) inches in diameter and larger,

(A) the diameter, and

(B) material type.

of the pipeline facility being located or carrier pipe, if applicable.

(Indiana Utility Regulatory Commission; 170 IAC 5-5-1.2)

## SECTION 4. 170 IAC 5-5-1.3 IS ADDED AS FOLLOWS:

## 170 IAC 5-5-1.3 Expiration of 811 Tickets

Authority: IC 8-1-26-26

Affected: IC 8-1-26-21, IC 8-1-26-16, IC 8-1-26-18

Sec. 1.3. (a) An 811 ticket expires at midnight, eastern standard time or eastern daylight savings time, as applicable in Greenwood, Indiana, twenty (20) calendar days after the date the notice is submitted to the association under IC 8-1-26-16.

(b) A person responsible for the excavation or demolition may not commence or continue work after calling in a re-mark ticket until the operators of pipeline facilities supply the information required by IC 8-1-26-18, or after the elapse of two full working days, whichever is later.

(Indiana Utility Regulatory Commission; 170 IAC 5-5-1.3)

## SECTION 5. 170 IAC 5-5-2 IS AMENDED TO READ AS FOLLOWS:

## 170 IAC 5-5-2 Notification of violations of IC 8-1-26

Authority: IC 8-1-26-26

Affected: IC 8-1-26-21

Sec. 2. (a) Any person or entity may An operator of a pipeline facility shall report a violation damage to a pipeline facility by providing the pipeline safety division with written or electronic notice of the violation damage within 21 days of becoming aware of the damage unless an extension is approved in writing by the director of the division.

(b) All violations shall be reported to the commission within thirty (30) days of a person becoming aware of the circumstances constituting the violation.

(b) An operator of a pipeline facility shall provide information about a damage within 21 days  after notice that a case number has been assigned by the pipeline safety division, unless an extension is approved in writing by the director of the division.

(c) a person may report a violation to the pipeline safety division by providing written or electronic notice of the circumstances giving rise to a violation.

(d) The pipeline safety division shall maintain, or cause to be maintained, a database of all reports provided to the pipeline safety division. (Indiana Utility Regulatory Commission; 170 IAC 5-5-2; filed May 25, 2011, 12:57 p.m.: 20110622-IR-170100184FRA; readopted filed Apr 11, 2017, 9:52 a.m.: 20170510-IR-170170124RFA)

## SECTION 6. 170 IAC 5-5-2.1 IS ADDED AS FOLLOWS:

## 170 IAC 5-5-2.1 Notice of inability to provide locate information

Authority: IC 8-1-26-26

Affected: IC 8-1-26-21

Sec. 2.1. (a) The notice under IC 8-1-26-18(k) shall be provided in one of the following ways:

(1) An oral communication in real time, either in person or electronically, with the person responsible for the excavation or demolition. The operator shall also make a written record, contemporaneously at the time of the oral communication, containing at a minimum:

(A) the date and time of the communication;

(B) the name of all parties that participated in the communication; and

(C) the new date by which the operator of the pipeline facility will provide the location information as required by IC 8-1-26-18.

A voicemail message is not sufficient notice under this subsection.

(2) A documented agreement between the operator and the person responsible for the excavation or demolition agreeing to a rescheduled deadline by which an operator will provide the location information as required under IC 8-1-26-18.

(b) An operator of a pipeline facility that does not provide notice as specified in this section:

(1) fails to notify the person responsible for the excavation under IC 8-1-26-18(k); and

(2) if IC 8-1-26-18(h) otherwise applies, the operator is subject to the penalty specified in IC 8-1-26-18(h).

(c) An operator complies with this section if it does all of the following:

(1) attempts to locate the facility at the site of the excavation but is unsuccessful because the facility is untoneable or for another documented reason;

(2) unsuccessfully attempts contact and creates a written record of the attempt as specified in subdivision (a)(1); and

(3) provides notice at the site of the excavation through marks on the ground or otherwise that the excavator may not dig because the area contains an unmarked facility.

(Indiana Utility Regulatory Commission; 170 IAC 5-5-2.1)

## SECTION 7. 170 IAC 5-5-2.2 IS ADDED AS FOLLOWS:

## 170 IAC 5-5-2.2 Clearance Zone

Authority: IC 8-1-26-26

Affected: IC 8-1-26-21

Sec. 2.2. As used in IC 8-26-20, “either side of the outer limits of the physical plant” means all outer limits of the physical plant, including above, below, and in a full radius surrounding the physical plant.

(Indiana Utility Regulatory Commission; 170 IAC 5-5-2.2)

## SECTION 8. 170 IAC 5-5-2.3 IS ADDED AS FOLLOWS:

## 170 IAC 5-5-2.3 Positive response to excavators of supplied location information

Authority: IC 8-1-26-26

Affected: IC 8-1-26-21

Sec. 2.3. (a) An operator of a pipeline facility shall provide an affirmative electronic response to the association in the manner directed by the association when the operator has provided the marking information in response to an 811 ticket.

(b) The association shall make available the operator’s response under subsection (a) to the person responsible for the excavation or demolition.

(c) An operator of a pipeline facility that does not provide the affirmative response required under subsection (a) violates IC 8-1-26-18(h)(1). If IC 8-1-26-18(h) otherwise applies, the operator is subject to the penalty specified in IC 8-1-26-18(h).

(Indiana Utility Regulatory Commission; 170 IAC 5-5-2.3)

## SECTION 9. 170 IAC 5-5-3 IS AMENDED TO READ AS FOLLOWS:

## 170 IAC 5-5-3 Process for determining violations of the underground plant protection laws and penalties

Authority: IC 8-1-26-26

Affected: IC 8-1-26

Sec. 3. (a) The pipeline safety division shall investigate alleged violations of IC 8-1-26.  an incident, whose addresses are reported or available on the internet, within sixty (60) days. The pipeline safety division shall investigate all incidents pursuant to applicable federal requirements and may investigate incidents further at its discretion.

(b) The pipeline safety division shall determine whether a violation of IC 8-1-26 occurred and by whom. After making a determination of a violation, the pipeline safety division shall forward its findings of violations of requirements provided in IC 8-1-26 to the advisory committee.

(c) The advisory committee will provide the person or entity accused of violating IC 8-1-26 with a copy of the pipeline safety division’s summary damage report.

(c) The advisory committee pipeline safety division will provide the person or entity accused of violating IC 8-1-26 with notice and an opportunity to appear before the advisory committee to provide written evidence prior to the advisory committee making a recommendation on the summary damage report.

(d) A person or entity accused of violating IC 8-1-26 may send correspondence and written evidence regarding the pipeline safety division’s finding of a violation to the advisory committee or in lieu of appearing appear at the public meeting in person to provide the written evidence. All correspondence must be addressed to the Underground Plant Protection Advisory Committee, Indiana Utility Regulatory Commission, 101 West Washington Street, Suite 1500 E, Indianapolis, Indiana 46204.

(e) The advisory committee shall use a publicly available penalty schedule when determining recommended penalties.

(f) Upon receiving a recommendation from the advisory committee, the commission shall provide the person or entity accused of violating IC 8-1-26 with notice of the advisory committee’s recommendation and provide the person or entity thirty (30) days to request a public hearing on the advisory committee’s recommendation.

(gg) A request for a public hearing before the commission must be in writing and shall be considered filed upon receipt by the commission filed and served following the procedures in 170 IAC 1-1.1.

(hh) If a person or entity accused of violating IC 8-1-26 does not request a public hearing, the commission willmay act upon the advisory committee’s recommendation.

(ii) All hearings before the commission regarding violations of IC 8-1-26 are subject to the commission’s rules of practice and procedure.

(j) A person or entity found in violation of IC 8-1-26 may be required to pay a civil penalty. All civil penalties must be paid to the commission within ninety (90) days of being assessed. (Indiana Utility Regulatory Commission; 170 IAC 5-5-3; filed May 25, 2011, 12:57 p.m.: 20110622-IR-170100184FRA; readopted filed Apr 11, 2017, 9:52 a.m.: 20170510-IR-170170124RFA)

## SECTION 10. 170 IAC 5-5-4 IS AMENDED TO READ AS FOLLOWS:

## 170 IAC 5-5-4 Underground plant protection account

Authority: IC 8-1-26-26

Affected: IC 8-1-26-18; IC 8-1-26-25

Sec. 4. (a) The commission shall administer the account.

(b) The account consists of civil penalties issued pursuant to IC 8-1-26.

(c) The commission shall maintain a publicly available website with information about the account.

(d) A person may submit a request for a grant to the commission that:

(1) complies with the purposes of the account under IC 8-1-26-24; and

(2) follows the guidelines set forth on the commission’s website.

(Indiana Utility Regulatory Commission; 170 IAC 5-5-4; filed May 25, 2011, 12:57 p.m.: 20110622-IR-170100184FRA; readopted filed Apr 11, 2017, 9:52 a.m.: 20170510-IR-170170124RFA)